Odin Offshore Safety Manual

July 2026 – NSGYR

- Table of Contents

1. Table of Contents	2

2. Pre-Race Crew Safety Briefing	4

2.1 Crew Briefing Checklist	4

3. Safety Equipment	5

3.1 PFDs & Tethers	5

3.1.1.PFDs	5

3.1.2. Tethers	5

3.1.3. Storage of PFDs	6

3.2. Personal Safety equipment - Mandatory	6

3.3. Personal items - Recommended	7

3.3 Grab Bag(s)	8

Life raft emergency equipment	8

3.4 MOB Safety Bag (rear rail)	8

4. Radio Scheds	9

5. Emergencies	10

5.1 General Guidelines	10

5.2 Man Overboard (also see Section 9)	10

5.2.1. Up Wind	11

5.2.2 Downwind.	12

5.3 Abandoning the vessel and deploying the life raft 
(also see 9.13 Abandon Ship)	12

5.4 Medical emergency (also see 9.3 Major Injury)	13

6. Crew organisation	13

6.1 Muster Numbers - ALL CREW ON DECK POSITIONS	13

6.2 Watches	14

6.3 Watch Handover	15

6.4 Routine Watch Duties	15

7. Ships Equipment	17

8. Heavy Weather	20

8.1 Heavy Weather Awareness	20

8.2 Heavy Weather Preparation	20

8.3 Heavy Weather Management	21

9. Standard Safety Procedures	22

9.1 MOB (also see Section 5.2)	22

9.1.1 MOB Prevention	22

9.1.2 MOB Procedure	22

9.1.3 If You Are MOB	23

9.2 Hypothermia	23

9.3 Major Injury / Life Threatening Situation	23

9.4 Fire at Sea	24

9.4.1 Fire - Precautions	24

9.4.2 Fire - Actions	24

9.5 Collision	25

9.6 Flooding	25

9.7 Dismasting	25

9.8 Grounding	26

9.10 Emergency Steering	26

9.11 Capsized	26

9.12 Towing	26

9.13 Abandon Ship	27

9.13.1 Launch Liferaft	27

9.13.2 Liferaft After Launch	28

9.14 Emergency Radio Communication	28

9.14.1 MAY DAY	28

APPENDIX 1 - GRAB BAG CONTENTS CHECKLISTS	29

APPENDIX 2 - RACE SPECIFIC CREW ROLES & MUSTER NUMBERS	31

APPENDIX 3 - RACE SPECIFIC WATCH SYSTEM	35

# 2. Pre-Race Crew Safety Briefing

Matt and the leadership team place the highest priority on crew and boat safety in all races on board. There will be a compulsory pre-race safety briefing on board the boat along with pre-race training days focusing on safety procedures.

Please read the following and acknowledge that you have understood each issue. Please do not hesitate to ask any questions.

# 2.1 Crew Briefing Checklist

| Item | Check |
| --- | --- |
| - Race Information - Description of the Course - Weather forecast - Safety checklist - ‘Sked’ details |  |
| - Safety Equipment |  |
| - Radio Scheds |  |
| - Emergencies |  |
| - Crew Organisation - Crew positions / numbers / buddies - Watches and watch handovers and procedures / chain of command - Emergency stations |  |
| - Ship’s Equipment |  |
| - Heavy Weather Procedures |  |
| - Standard Safety Procedures |  |

# 3. Safety Equipment

## 3.1 PFDs & Tethers

### 3.1.1.PFDs

All crew on deck must wear PFD’s that satisfy Australian Standards in conditions of “heightened risk” during offshore races. This includes one hour before sunset until sunrise and at any other times where the designated skipper or watch captain considers there is heightened risk. You may, of course, wear a PFD at any other time you choose.

Odin will carry 12 inflatable PFDs for all racing.

If you are using your own PFD please ensure you confirm so we only carry what is required.

- You are responsible for your own PFD maintenance and compliance to sailing criteria. Please ensure you have carried out your pre-race check on your lifejacket, which must be tried on and fitted (including crotch strap) prior to the commencement of the race. CYCA requires all lifejackets to be serviced by a recognised service professional and for a certificate to be provided. You are responsible for ensuring your own lifejacket carries a valid certificate.

- For offshore racing, vests now need a splash hood.

As a rule, No Exceptions… - Everyone wears a lifejacket from sunset to sunrise regardless of conditions. - Lifejackets to be worn upwind in a consistent breeze above 18 knots 1st reef - Lifejackets worn when sailing downwind with the A4 hoisted or a consistent breeze above 18 knots. Tethering depends on conditions during daylight hours and is compulsory at night. - Lifejackets must be worn if instructed by the skipper or Watch Captain.

### 3.1.2. Tethers

Tethers on board Odin conform to the latest SI and guidelines from AS. If you wish to use your own tether, please ensure this is compliant with the latest recommendations and SIs and discuss this with the skipper.

### 3.1.3. Storage of PFDs

Each crew member will be assigned to a tag number used for the purpose of storing your PFD when not in use. These will be marked 1-12 in the starboard head. The number allocated will be the same as your muster number. There will be 2 unmarked spare jackets. For clarity, wet weather gear goes on your hook and your PFD and boots go in the buckets.

PFDs are not to be stored anywhere else on board unless off watch at night and you choose to have it with you in your bunk.

In the event of a catastrophic failure PFDs will be retrieved along with the Grab bags and thrown on deck.

## 3.2. Personal Safety equipment - Mandatory

- Each crew member must ensure that they have a GPS PLB and an AIS Transponder 
(PLB - Personal Locator Beacon) (AIS - Automatic Identification System) 
(both not supplied by the owner).

  - The PLB Hex ID must be updated in your personal file on Top Yacht / SailSys and updated for every Cat 1 or 2 race on the AMSA beacon website https://www.beacons.amsa.gov.au/.

  - The AIS must be programmed into the boat computer so that it can link to the boat’s receiver and we can come and find you if you are MOB.

- Each crew member must have a PFD (Personal Flotation Device) - rated 150N buoyancy (minimum) and compliant with ISO 12402-3 or AS 4758. Your PFD must have been professionally serviced in the past year and must have a certificate to prove it. It is essential that PFD’s supplied by crew and owner are fitted with a strobe light, whistle, crotch straps and a storm hood.

- Each crew member shall ensure that they have a safety tether in full working order
(this can be supplied by owner - please advise if you require one).

- Each crew member shall ensure they have a personal head torch and a cyalume stick (glow stick)  (not supplied by owner). Please ensure your torch has a red light for use at night and spare batteries. Purchase of a small bumbag is advisable. It can be easily worn 24/7 with your PLB, torch, cylumes and dye pack.

- Personal knife or ‘leatherman’

## 3.3. Personal items - Recommended

- For offshore racing, personal equipment bag volume to be limited to (20L)

- All personal effects to be stored within your personal bag in designated location

- Valuables to be stored either in your personal bag or if requested can be placed into the “Navigation (valuables) grab bag” at the Navigation Station

- Foul weather clothing (ideally not black or white)

- Thermals

- Sea Boots or waterproof socks with sailing shoes

- Bum bag that contains:

  - Sea marker dye

  - Whistle

  - Laser Flare

  - PLB

  - Cylume (glow) stick

  - Swim cap

  - Goggles (optional)

- Cap or hat

- Beanie

- Change of socks and undies

## 3.3 Grab Bag(s)

There are 2 types of grab bags on board Odin:

- Navigation Position Grab Bag at the navigation station

- Main Liferaft Grab Bags (2 x No.) stored cockpit flooring under hatch

Checklists for the contents of each type of Grab Bag are listed in Appendix 1

### Life raft emergency equipment

In addition to the grab bags, standard equipment packed in the life rafts include:

- Torch, batteries, bulb

- Signal mirror

- Seasickness bags

- Seasickness pills

- Hand flares x 6

- Parachute flares x 2

- Whistle

- Thermal protective aid

- Drinking water 72 x 125ml

- Food rations x 6

- First aid kit (AMSA/SOLAS)

- Lifesaving signal card

- Sunscreen

- EPIRB

## 3.4 MOB Safety Bag (rear rail)

A dedicated MOB Safety Bag will be slung on the rear rail for use in an MOB emergency.

It will contain:

- Floating ‘dolphin’ torch (to throw to the MOB)

- Cylume (glow) sticks (for throwing in the water as a trail)

- Dye marker

- Powerful waterproof ‘search’ torch

- Throwing line

Also at the rear rail is a JonBuoy Danbuoy that can be deployed immediately by unclipping the black retainer strap and throwing the unit overboard. It will automatically inflate and deploy. This should be done as a priority in the sequence of MOB.

# 4. Radio Scheds

Daily position reports will be conducted by Satellite Phone at 1835 hours on 25 July 2026 followed by 0635 hours and 1835 hours on each subsequent day; and at other times as may be advised by Race Control.  A weather report may be issued at this time.

All communications for the start shall be conducted on VHF Channel 72

Continuous watch on VHF CH 16

“CYCA RACE CONTROL” comprises Marine Rescue Sydney and Race Management - +61 488 846 885.  “CYCA RACE CONTROL” will conduct position reports and be available via phone for the duration of the race. “CYCA RACE CONTROL” will assist boats in distress by relaying traffic to the appropriate authorities.  “CYCA RACE CONTROL” is not intended to relay private messages, except when deemed necessary and under RRS 1.1 Rendering Assistance.

For unscheduled communications, boats should call “CYCA RACE CONTROL” on +61 488 846 885. “CYCA RACE CONTROL” (as monitored by Marine Rescue Sydney) will maintain a continuous listening watch on VHF Channel 16 at all times. The Finish Line will maintain a listening watch on VHF 72 ONLY.

Satellite Phone

OUR SAT PHONE NUMBER IS 0450 990 009- This will be turned on in duration of the race and is the number used for comms for the race office position reports and calls.

In an emergency there is an additional Emergency Sat Phone number for the device in the Nav Station Grab Bag which is 0424 210 319

Wifi

Please also note that Odin is equipped with Star Link and “voice over data” calls can be made from any device logged on to the boat Wifi

Wifi – ODIN

Password – First5353

# 5. Emergencies

## 5.1 General Guidelines

Emergencies come in various guises and are rapidly evolving scenarios. In addition to “MOB” (man overboard) and “abandon ship” detailed below, this encompasses the steps to be taken by crew in the event of ANY emergency. Further Standard Safety Procedures are documented in Section 9.

- Emergency call by Watch Captain “EMERGENCY! ALL HANDS ON DECK”

- Crew to assume their “all crew on deck position”

- Skipper or Person In Charge to assume command of the emergency situation

- Summary of events to be transmitted to the Person In Charge

  - The Watch Captain or the Watch Captain nominated person to handover.

No other discussion should occur on deck until the end of the handover.

- Use communication format ISAR

  - Introduction = nature of emergency

  - Situation = current situation

  - Assessment = main task that needs addressing

  - Recommendation = suggestion of next step

- Person In Charge to direct crew members to respond to the emergency.
No crew member to leave their post until directed to do so by the Person In Charge

- Contact with race control / general urgency or emergency call to be made by radio operator at the direction of Person In Charge.

EMERGENCY Chain of Command – will be defined prior to every race.

As a guideline for the offshore season 2026:

- Matt is the Skipper and is the ultimate Person In Charge

- Watch Captain or Andy will become the Person In Charge if Matt is not available or if this is delegated by the Skipper.

- The Primary Radio Operator will be responsible for radio contact and communication during an emergency. If the Primary Radio Operator is not available the Secondary Radio Operator will assume this role.

## 5.2 Man Overboard (also see Section 9)

When a crew person is overboard, the boat will be stopped as soon as possible and when safe, commence a course back to the crew person for retrieval.

For all MOB situations the person that sighted the MOB SHOUTS MAN OVERBOARD until acknowledged by the helm and crew on deck. Call below for the offwatch “ALL HANDS ON DECK MAN OVERBOARD”

### 5.2.1. Up Wind

1. An appropriately well positioned person, other than the helms person will immediately throw the Life ring or Danbuoy towards the person and then point to the crew person overboard and commence counting aloud.

This person shall not stop looking and pointing at the crew person overboard under any circumstances. At night the nearest torch (alight) and glow sticks (broken) are to be thrown by a 3rd crew person, in the direction of the overboard crew at intervals of 1 minute. A ‘Dolphin’ torch and glow sticks and heaving line will be stowed in the MOB Safety Bag on the rear lifeline/rail for this purpose. A powerful waterproof torch for search purposes will also be located in this bag.

2. The man overboard button located at The Nav station and or the steering console shall be pressed as soon as possible by the closest person. There shall be a person at the navigation table (primary or secondary navigator who is NOT on watch) until relieved by a radio operator.

3. The helms person will immediately sail a close hauled course to reduce speed, luffing the jib until it can be dropped and place the throttle in neutral position to allow the engine to be started. Drop the jib as soon as possible and safe to do so.

4. The radio operator will commence a PAN PAN man overboard call on channel 16

5. Upwind, it is assumed that all lines are inboard and therefore the engine can be started immediately, once in neutral, however a visual check will be made to confirm. The helm will be responsible for starting the engine only when given the all-clear for ropes and obstructions.

6. Once the engine has started, the mainsail can be dropped to deck as soon as the pit person and other crew are ready. If it is suitable to do so.

7. Once the main has been dropped to the deck, the boat can immediately complete a 180 degree turn to the direction of the person overboard.

8. Once the turn has completed the crew person pointing towards the last known crew overboard position, re-commences counting.

9. Once the crew overboard has been located, the helms person shall approach the crew person dead upwind then to windward.

10. Retrieval shall be completed with a sling attached to a retrieval line being the main halyard or by another recovery method as directed by the Person In Charge. 

If the MOB person is unconscious, a second nominated person in harness is to be lowered with the sling as directed by the Person In Charge.

### 5.2.2 Downwind.

1. An appropriately well positioned person, other than the helms person will immediately throw the life ring or danbuoy towards the person and then point to the crew person overboard and commence counting aloud. This person shall not stop looking and pointing at the crew person overboard under any circumstances. 

At night the nearest torch (alight) and glow sticks (broken) are to be thrown by a 3rd crew person, in the direction of the overboard crew at intervals of 1 minute. Glow sticks, ‘dolphin’ torch  and heaving line will be stowed in the stern bag. A powerful search torch will be located in the MOB Safety Bag on the rear rail/lifelines.

2. The man overboard button on the main nav station panel/steering console shall be pressed as soon as possible by the closest person. 

3. There shall be a person at the navigation table until relieved by a radio operator, who will commence a PAN PAN call on Channel 16.

4. The spinnaker trim will immediately release the sheet to collapse the spinnaker. The main should be brought to the centre line if possible, to slow the boat. If the A-sail is furling it shall be furled asap. If it is a non-furling A-sail, the crew will prepare to manually retrieve the kite behind the main and into the aft hatch.

5. The tack line must remain on until the spinnaker is well advanced into the boat.

6. The halyard / tack line will be released as soon as possible by the pit person having regard to crew being in position for spinnaker retrieval.

7. Once the spinnaker is inboard, the helm can immediately go head to wind, close hauled.

8. The skipper or Person In Charge, trimmers and bow team shall conduct an inspection to ensure all lines are inboard.

9. Once the all clear signal has been given, and not until it is given, the engine can be started.

10. Once the engine has been started, the main can be dropped to deck and a course back to the crew person overboard can be commenced and retrieval as described above.

## 5.3 Abandoning the vessel and deploying the life raft 
(also see 9.13 Abandon Ship)

If required to abandon ship we will muster to the stern of the boat and await the deployment of the life rafts. The Skipper will make the final decision to abandon ship.

We carry 2 rafts and it's essential that everyone follows the direction of the Person In Charge of the emergency (see above).

In the event of a controlled abandoning of the vessel, here are the allocated roles:

- Skipper to coordinate as Person In Charge of the emergency process of abandoning ship. The Skipper may nominate another crew member as Person in Charge at his discretion.

- 2 x nominated individuals will be responsible for the launch of the rafts.

- The Primary Radio Operator will be in charge of Radio comms.

- The Navigator will collect the Navigator’s grab bag and any PFD’s below deck - everyone will wear their PFD.

- Person In Charge to ensure that everyone is accounted for by coordinating a muster call. Remember your number and the person either side of you.

For this race all our wallets and phones will be stored in another 10L dry bag which will be inside the Grab Bag. If we need to leave the boat in a sudden emergency, whoever is closest at the time will retrieve it.

The decision will be made by the Skipper or Person In Charge to abandon ship into the life rafts.

## 5.4 Medical emergency (also see 9.3 Major Injury)

Medical emergencies are diverse but injuries probably predominate on board any offshore sailing vessel. The response to a medical emergency will vary depending on circumstances.

General principle:

- Watch Captain assigns one member to attend/assess injury

- If more help or uncertainty around the severity of the event is required, Watch Captain will put out “ALL HANDS ON DECK CALL”.

- The first aid officer on watch will make an initial assessment of the medical event

- If indicated, the patient is to be moved to “sick bay” – first bunk on starboard side in the centre of the cabin.

- If required, additional medical assistance / treatment can be sought from the most senior medical officer. For significant injuries the Skipper and Crew Boss should be notified.

- Radio operator to contact race control / put out emergency call if needed.

# 6. Crew organisation

## 6.1 Muster Numbers - ALL CREW ON DECK POSITIONS

A “Number Off List” is attached which also should correspond with your PFD number and your locker number. Take some time to know your number and the person either side of you. You will be quizzed on it.  Refer to APPENDIX 2 - CREW ROLES for race specific role allocations.

| # | NAME | Role | Specialist Roles | All Hands on deck POSITION |
| --- | --- | --- | --- | --- |
| 1 |  | Skipper |  | Helm |
| 2 |  | Sailing Master/ Helm |  | Stern starboard side |
| 3 |  | Mainsheet |  | At mainsheet Pod |
| 4 |  | Pit |  | Cockpit (port at winch) |
| 5 |  | Pit |  | Cockpit (starboard at winch) |
| 6 |  | Trim |  | Cockpit (port mid cockpit) |
| 7 |  | Trim |  | Cockpit (starboard mid cockpit) |
| 8 |  | Mast |  | Cockpit (port mid cockpit) |
| 9 |  | Mast |  | Cockpit (starboard mid cockpit) |
| 10 |  | Bow |  | Cockpit (port against coach house) |
| 11 |  | Bow |  | Cockpit (starboard against coach house) |
| 12 |  | Navigator |  | Stern port side |

## 6.2 Watches

- There will be a watch system posted to the boat prior to the start of the race’.

- Each will be commanded by a Watch Captain and will include 5/6 crew members with Navigator floating.

- The on watch shall wake the ‘off’ watch 20 minutes before the start of their duty.

- If desired, coffee or tea can be provided by the offgoing watch according to individual posted preferences.

- A "lifejacket / harness & hooked on” policy between 1800 - 0600 (sunset to sunrise) is to be strictly observed with them being worn during the night regardless of the sea conditions and wind conditions above 25 knots.

- Those requirements may be expanded upon but not reduced at the discretion of the Watch Captain.

- In the event of an incident, all hands may be called on deck. Should this happen, crew  are to make their way on deck as they are during daytime, at nighttime with life jackets and harness on and await further instructions.

- Only on watch crew are to leave the cockpit area to go forward or secure sails etc with correct safety gear fitted.

Refer to APPENDIX 3 - RACE WATCH SYSTEM

## 6.3 Watch Handover

On the hand-over of the watch, the off-going Watch Captain is to brief the incoming Watch Captain of the following:

- Sail configuration

- Wind direction and strength

- Any apparent trends in the weather pattern

- Details of any ships lights or land in sight

- When the last deck check was done

- Any outstanding tasks to be performed

- Any forecasted event detailed by the skipper

- When and if the Skipper or Navigator is to be called

- Any unusual circumstances which may impact the incoming Watch Captain

- Any other detail deemed appropriate

## 6.4 Routine Watch Duties

- A thorough inspection is to be made of the yacht's rig, fittings, spars and sails, sheets and structure (above decks) each hour (depending upon the prevailing conditions). Particular attention should be paid to the chafing points on the sails and sheets, to loose shackle pins and to signs of failure in the standing rigging. At night, this inspection is to be conducted using a suitable powerful torch.

- On assuming the watch, a thorough inspection of the below deck is to be made including the bilge for water, oil or other foreign matter. Careful note should be taken of the position of the heads sea-cocks.

- Any gear likely to move should the yacht encounter rough seas is to be stowed correctly in its allocated position. Wet weather gear is not to be left on the cabin sole and must be stored in the wet locker.

- Sheets are to be maintained in a neat and orderly fashion so they are readily available for use and do not contribute towards a safety hazard for the crew.

- Remember, we are racing at all times.

- Members using the galley are to clean up after themselves. Rinse coffee mugs and any bowls or cutlery and stow their equipment in the shelves

- The Navigator is to be called immediately if any unusual event occurs whether it directly affects the yacht or not.

Notifiable events include:

- Significant changes in the weather

- A crew member is injured

- Any damage is sustained to the yacht

- Any new and significant lights are spotted, or

- A distress or storm warning message is received

## 7. Ships Equipment

Crew kit and storage

- life jackets, tethers, foul weather in starboard head/shower on numbered peg and in buckets

- personal dry bag in forward cabin

Boat position

- available on GPS plotter at Nav station and at helm

- Paper charts and position to be maintained

- Ship’s log

VHF Radio

- Starboard side near dining table

- emergency antenna in starboard middle ‘sofa’ locker

- Laminated emergency numbers by side of radio

- Handheld VHF in Nav Grab Bag and on the deck next to the MOB bag

Satellite phone

- In Nav grab bag

Radio

- Mayday call script laminated by VHF radio

- Emergency stations also on laminated sheet

- DSC button on VHF and handheld VHF in both grab bags

Flooding

- Manual bilge pump port rear cabin locker

- Use toilet with hose and vacuum (soccer ball)?

- Buckets in starboard rear quarter hatch

- Keel bolts accessible by lifting cover

- Through hull fittings:

  - Sea cocks below sink, in each head/shower, engine, cockpit drain, and cockpit floor locker drain

  - Log under floor starboard forward cabin

  - Rudder through tender garage

  - Sail drive

Fire

- Battery switches starboard side cupboard (opposite galley)

- Fuel switch off at starboard aft bunk by fuel tank

- Gas switch off in anchor locker at bow – isolator in the galley cupboard

- Fire extinguishers located in forward and rear port cabins

- Fire blanket by stove

Dismasting

- Angle grinder in stbd ‘seat’ locker at galley table

- bolt cutter in port bunk lower compartment

- Bosuns chair in cupboard opposite head

First Aid

- First aid kit port side in rear seat lockers

Sail Plan

- Sail selection plan laminated outside near instruments

Steering Failure

- Emergency tiller in dinghy locker

- Access to steering cable through dinghy locker

- Drogue and spare anchor in centre locker access from rear starboard cabin

Flares

- Port midship under ‘seat’ locker at galley table and in the grab bags

Man Over Board

- MOB button on GPS plotter below and at helm

- Danbuoy, sling, life ring all on stern rail

- Glowsticks, dye, flashlights and heaving line all in MOB Safety Bag on rear rail/lifelines

- Swimmer of the watch mask, fins starboard stern quarter hatch

Evacuation

- Life rafts on rear deck

- Hydrostatic release

- Grab bags in cockpit flooring hatch

- Nav Grab Bag hanging next to stairs

# 8. Heavy Weather

## 8.1 Heavy Weather Awareness

Weather information

- channel 67 (3 minutes before the hour)

- Automated weather 4 hourly from VMC and VMW

- GRIBS over sat phone

- Daily sked

Visual, heads out of the boat

Barometer

- 1 mb/hour – likely force 6 (22-27 knots)

- 2 mb/hour – likely force 7-8 (28-40 knots)

- 3 mb/hour – likely gale force exceeding force 8 (40+ knots)

Beaufort Wind / Sea Scale

| Beaufort | Wind Speed | Description | Wave Height |
| --- | --- | --- | --- |
| Force 6 | 22-27 kts | Strong Breeze | 3m |
| Force 7 | 28-33 kts | Near Gale | 4m |
| Force 8 | 34-40 kts | Gale | 5.5m |
| Force 9 | 41-47 kts | Strong Gale | 7m |
| Force 10 | 48-55 kts | Storm | 9m |
| Force 11 | 56-63 kts | Violent Storm | 11.5m |
| Force 12 | 64+ kts | Hurricane Force | 14+m |

### 8.2 Heavy Weather Preparation

Clear decks and drain points

Secure below decks - Imagine vessel upside down

Engineering check - Charge battery - Test pumps

Navigation - Confirm GPS position, mark charts - Radio position to coast station

Crew - Consider food and drink (eat early before bad weather) - Sea sickness pills for those that need / everyone - Life jackets, tethers, goggles, thermals, personal safety equipment, foul weather gear - Restructure watch systems if necessary - No-one to go forward without approval. In bad weather consider minimal crew on deck

Sail selection - Ready storm sails - Reef early - Spare sheets and rig backup sheets

### 8.3 Heavy Weather Management

Handling Strategy: - Run / Windward / Reach / Bare poles vs storm jib / Drogue / Engine?

Regular check: - Steering - Keel bolts - Toilet - Water below - Chafe on sails, halyards, sheets (run backup sheets x 2) - Leech flutter - Reefing strops - Deck fittings

Maintain navigation

Minimise stress on boat (storm sails, point of steering)

Ongoing assessment: - Crew - Duration of storm - Tide/current direction and strength - Viability of seeking shelter - Navigation options

# 9. Standard Safety Procedures

## 9.1 MOB (also see Section 5.2)

### 9.1.1 MOB Prevention

- Use a lifejacket / harness in rough weather and at night.

- Make sure PFDs are adjusted to a tight fit or you can fall out of them.

- Wear suitable protective clothing.

- Jackstays are rigged on both sides of the boat so that you can walk the full length of the deck without having to unclip.

- Harness attachment points are close to the companionway so that you can clip on before coming on deck and on both sides of the cockpit.

- Clip on when steering in rough weather.

- Some surfaces are slippery even with stick on non slip strips. Pay particular attention to the tops of hatches and sloping coach roof sides which become walkways when the boat is heeled.

- Guardrails are continuous around the deck. Some ends on the transom are secured with quick release clips so that you can release them to recover a person from the water.

### 9.1.2 MOB Procedure

- Yell “Man Overboard”; commence pointing and do NOT look away even momentarily

- If multiple spotters, then Helm to nominate single or double spotters to continue

- Press MOB button at Helm Station

- Throw life ring, sling, danbuoy, dye, glow-sticks, flashlight turned on (as appropriate)

- Navigators note course, wind; (at 6-8 knots, 3-4 m/s, 10 sec = 30-40m)

- If upwind, immediately heave to - tack but do not tack the jib

- If downwind, steer down wind and depower (ease sheet and centre main)

- Bring up any crew below decks

- Turn batteries on, consider engines on

- Tack around

- Swimmer of the watch prepares to swim, wearing a life jacket but not inflated, consider wet suit. Retrieval line on deck and attached to a hard point

- Bring boat alongside

- If MOB unconscious/injured, swimmer of the watch to go in (suitably tied on)

- If MOB conscious, lasso with heaving line

- Consider retrieval depending on sea state

- Transom recovery using swim platform?

- Recovery using halyard affixed by MOB

- Recovery using crew member on halyard taking down second halyard?

### 9.1.3 If You Are MOB

- Look for the lifebuoy which may be close by.

- Remain calm, keep your legs close together and restrict your movements to stop flushing cold water under your clothing.

- Whatever your situation, conserve your body heat - the greatest threat to your survival is from the cold.

- Remember in cold water during the winter your ability to assist in your rescue will be greatly diminished after 10 minutes.

- In rough conditions, turn your back to the waves to keep your mouth and nose clear of spray. Pull your spray hood over if you have one.

- Tighten up wrist, ankle and neck fastenings of protective clothing to reduce heat loss and the onset of hypothermia.

- Do not attempt to swim back to the boat for the same reasons.

## 9.2 Hypothermia

- Minimise movement, handle gently

- Shelter victim from wind and cold

- Keep victim horizontal

- Look, listen and feel for breathing

- Check pulse neck for at least 45 sec

- Apply warmth to groin, neck, head and chest

- Do not massage skin

- Avoid direct contact with a strong heat source; focus on warm water bottle, warm air, body heat transfer

- Replace wet clothes with dry

- Warm environment (stove, engine)

- Huddle / spoon (skin to skin best)

- Blankets and head covering

- Warm sweet drinks if conscious

## 9.3 Major Injury / Life Threatening Situation

- A life-threatening situation is if a member of the crew is deemed to have been seriously injured, and outside assistance is required.

- Consider Pan Pan call using VHF Channel 16 notifying a shore base and other vessels who may have a qualified doctor on board.

- Attend victim and follow first aid procedures.

- Organise rest of crew to slow boat and determine the closest/safest port.

- Alter course to sail towards help

- Consider the option of a medical evacuation by boat or helicopter

## 9.4 Fire at Sea

### 9.4.1 Fire - Precautions

Fire at sea is one of the most terrifying and potentially devastating situations that can be encountered by a ship's crew.

Fire precautions are to be observed:

- Smoking is to be restricted to the cockpit aft of the wheels only.

- When sailing downwind extreme caution must be taken to ensure that no hot ash is allowed to blow onto the sails, or below.

- Any fuel-contaminated material is to be thoroughly rinsed and dried before storing.

- The  stove gas on I off procedure must be strictly adhered to.

### 9.4.2 Fire - Actions

- Raise the alarm “Fire, Fire, Fire”

- Secure vessel, survival equipment, crew and passengers

- Assess fire

- Communicate best plan

- “Fight or flight”?

- Train, know where the equipment is and how to use it/ read instructions

#### Fire fighting

- Switch off 12V breakers / main battery switch if electrical fire

- Switch fuel shut off valve to OFF if engine fire

- Switch gas shut off valve to OFF and disconnect gas bottle if galley fire

- Commence fighting the fire with the most available means (as long as safely available)

- Minimise oxygen

- Remove or control fuel

- Cool boundaries

- Buckets

- Fire extinguishers

- Fire blankets

- Minimise wind across decks

- Manoeuvre to keep flames and smoke outboard

## 9.5 Collision

- A collision will be avoided at all costs. To this end, at night, a powerful torch and a white flare will be kept handy to alert ships at sea of the presence of the yacht.

- If however, a collision appears imminent, the helmsman is to call "PREPARE FOR COLLISION" or words to that effect. On hearing this other crew members are to brace them in such a position as close as possible to the yachts centre line and wait.

- If a collision subsequently occurs, members should make their way to the deck noting any damage on the way.

- Members on deck should, immediately following the collision, commence a head count to ensure that all are accounted for.

- In the event of an unseen collision i.e. object under the surface of the water, the following persons will be responsible:

  - Watch Captains or 2IC to Seek out area of damage & assess

  - Upon order of the Skipper prepare to abandon ship

## 9.6 Flooding

- Raise the alarm

- Secure the vessel

- Assess damage

- Choose and communicate plan

- Control / limit damage

- What is the source?

- Adjust course, heel and speed to minimise ingress

- Use all pumps and buckets

- Shore up internally – wedges, plugs, cushions, sails

- Shore up externally – sails

## 9.7 Dismasting

- Watch Captain to raise the alarm but hold on numbers on deck

- Assess damage esp. to hull integrity

- Consider pan pan

- Person In Charge to control operations and assign tasks

- Secure the boom with main sheet (in)

- Consider jettison rig or save parts

- Hacksaw Blades to be retrieved from the locker, spanners from toolbox.

- Retrievable spars to be identified

- Before wire rigging are cut away, the Watch Captain will assess the situation with the skipper to determine the "cut away" points.

- Use grinder to cut rods.

- Cut away mast side first, then forward then upside

Note: It may not be necessary to cut away the entire rig i.e. if the lower half of the mast is still standing then leave it, OR if a straight piece of mast can be saved then this should be done. This is in order that a jury sail can still be positioned.

## 9.8 Grounding

- Assess damage

- Secure vessel

- If stranded

  - MAYDAY

  - EPIRB

- If need to evacuate then

  - Gather equipment

  - Grab-bags

  - Deploy life-raft

## 9.10 Emergency Steering

- Raise the alarm

- Secure the vessel

- Assess damage

- Plan and Proceed

- Damaged steering – emergency tiller

- Damaged/lost rudder – adjustable drogue, spinnaker pole with board, steer with sails

## 9.11 Capsized

Survival considerations

- Safety equipment location and access when upside down

- Stowage of water and food

- Escape hatch positions

If capsized

- Headcount

- Injuries

- Rig threat

- Hull integrity

Critical assessment

- Remaining assets

- Communications

## 9.12 Towing

- Long (wave spacing) and strong line

- Bridle

- Strong points

- Use chain or anchor to weigh down

- Emergency release mechanism

- Consider trim, drogue, speed, communications, chafe, lights

## 9.13 Abandon Ship

- Made by Skipper or delegated Person In Charge

  - Yacht is lost (sinking or fire)

  - Abandonment equipment and processes are becoming threatened

  - Yacht is uninhabitable

- Controlled abandonment

  - Raise the Alarm

    - Navigator to determine position

    - Send Mayday

    - Activate EPIRB

    - Set off flare

- Secure the Boat

  - Stop the engine

- Assess and communicate

  - Crew to muster stations with lifejackets donned and  grab-bags

  - Headcount (number off)

- Act

  - Prepare and launch life raft

  - On no account is anyone to unlash or deploy the Life raft before being directed to do so by the Skipper.

### 9.13.1 Launch Liferaft

The Skipper (or delegated Person In Charge) will detail all action appropriate to the abandon ship situation. If any circumstance is believed to be unknown to the Skipper, he should be advised. Otherwise, all orders are to be actioned immediately. Talking should be kept to a minimum with only that conversation necessary to confirm detail. This is to ensure three things:

- Most effective communication without confusion

- Minimal panic

- Any unusual noises emitted from the yacht may be heard

- If abandonment is ordered at night, individuals are to immediately ignite their personal torches and all the yachts lights are to be turned on.

- There are 2 x 6 man life rafts both equipped with Flares and EPIRBS

- Launch areas clear

- Extra provisions assembled

- Life raft lashings removed

- PAINTER SECURED TO VESSEL

- Area clear of fittings and railings

- Orderly embarkation

- Avoid entering water if possible

- Try to step into the life raft rather than jumping

### 9.13.2 Liferaft After Launch

- CUT painter close to yacht

- STREAM drogue

- CLOSE canopy

- MAINTAIN (raft, crew, rescue efforts)

- Launch the EPIRB

  - Seasickness pills

  - Urinate

  - Flares

  - PLB one or max 2 at a time

  - Radio calls

  - Lookout roster

  - Navigation

  - Morale

  - Ration planning on 3 x days in raft;  0.5L per person per day

## 9.14 Emergency Radio Communication

May Day

- A vessel or person is in grave and imminent danger

Pan Pan

- An urgent message concerning the safety of the vessel or a person (including MOB)

Securite

- An important navigational or weather warning

### 9.14.1 MAY DAY

- Send DSC message first on VHF, HF

- Channel 16 on VHF

  - Mayday, Mayday, Mayday

  - This is Odin, Odin, Odin

  - Mayday

  - Odin

  - Position (Lat, long)

  - Nature of problem

  - Assistance required

  - Number of persons aboard

  - Other boat identifying features, actions

  - Over

- Consider EPIRB activation

  - If activated and contact is made by radio, advise that EPIRB activated

  - Do not turn off until told to do so by rescue authorities

- Allow a short period for reply, if no answer repeat distress call on other frequencies

  - VHF 9, 16, 70

APPENDIX 1 - GRAB BAG CONTENTS CHECKLISTS

The Navigation Position Grab Bag (1 No.):

| Items | Checked |
| --- | --- |
| Spare handheld VHF (working and charged) | Confirmed |
| Satellite phone (working and charged) | Confirmed |
| Mobiles (separate dry box) | TBC |
| Main vessel EPIRB | Confirmed |
| Handheld compass | Confirmed |
| Wet/dry notebook | Confirmed |
| Pencil | Confirmed |

Located at the navigation station

The Main Liferaft Grab Bags x 2

Will be assembled by a nominated crew member before the race. They should be assembled on the instruction of the skipper and will include:

| Items | Checked |
| --- | --- |
| Dye | Confirmed |
| Cyalume light sticks | Confirmed |
| VHF Radio | Confirmed |
| Waterproof handheld GPS | Confirmed |
| Extra water - grab from the cockpit flooring hatch locker | Confirmed |
| Extra food - grab from the galley (if time available) | Confirmed |

MOB Safety Bag (rear rail):

A dedicated MOB Safety Bag will be slung on the rear rail for use in an MOB emergency.

It will contain:

| Items | Checked |
| --- | --- |
| Floating ‘dolphin’ torch (to throw to the MOB) | Confirmed |
| Cylume (glow) sticks (for throwing in the water as a trail) | Confirmed |
| Dye marker | Confirmed |
| Powerful waterproof ‘search’ torch | Confirmed |
| Throwing line | Confirmed |

Life raft emergency equipment

Standard equipment packed in the life rafts include:

| Items | Checked |
| --- | --- |
| Torch, batteries, bulb | Confirmed |
| Signal mirror | Confirmed |
| Seasickness bags | Confirmed |
| Seasickness pills | Confirmed |
| Hand flares x 6 | Confirmed |
| Parachute flares x 2 | Confirmed |
| Whistle | Confirmed |
| Thermal protective aid | Confirmed |
| Drinking water 72 x 125ml | Confirmed |
| Food rations x 6 | Confirmed |
| First aid kit (AMSA/SOLAS) | Confirmed |
| Lifesaving signal card | Confirmed |
| Sunscreen | Confirmed |
| EPIRB | Confirmed |

APPENDIX 2 - RACE SPECIFIC CREW ROLES & MUSTER NUMBERS

Muster Numbers - ALL CREW ON DECK POSITIONS

| # | Name | All Hands on deck Position |
| --- | --- | --- |
| 1 | Matt | Helm |
| 2 | Willy | Stern starboard side |
| 3 | Tom | At mainsheet Pod |
| 4 | Pete A | Cockpit (port at winch) |
| 5 | Katie | Cockpit (starboard at winch) |
| 6 | Nik | Cockpit (port mid cockpit) |
| 7 | Margot | Cockpit (starboard mid cockpit) |
| 8 | Simon | Cockpit (port mid cockpit) |
| 9 | Joris | Cockpit (starboard mid cockpit) |
| 10 | Charles | Cockpit (port against coach house) |
| 11 | Carpo | Cockpit (starboard against coach house) |
| 12 | Andy | Stern port side |

CREW ROLES

| Watch # | NAME | Role | Specialist Roles | Muster # |
| --- | --- | --- | --- | --- |
| 1 | Willy Roberts | Watch Captain/ Helm / trim | Launch liferaft # 1 | 2 |
|  | Simon Hanning | Trim/Steer | Primary First Aid | 8 |
|  | Katie O’Mara | Pit / Trim | Secondary Radio | 5 |
|  | Nik Payten | Mast/ Mid Bow | Swimmer of the Watch | 6 |
|  | Joris Meijboom | Bow/Trim |  | 9 |
| 2 | Tom Sloane | Watch Captain/ helm / trim | Secondary First Aid, Secondary Radio | 3 |
|  | Peter Austin | Trim / Steer | Swimmer of the Watch Launch Liferaft # 2 | 4 |
|  | Margot Van Der Poel | Pit / Trim | Secondary First Aid | 7 |
|  | Charles Russell | Mast/ Mid bow |  | 10 |
|  | Peter Carpenter | Bow/Trim |  | 11 |
| (3) Float | Andrew Forbes | 2IC Tactician/ Navigator/Steer | Primary Radio - Back up First Aid | 12 |
|  | Matt Hanning | Skipper/Steer | Secondary Radio | 1 |

# APPENDIX 3 - RACE SPECIFIC WATCH SYSTEM

| Watch System - Sydney / Gold Coast Race 2026 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Day 1 | Day 2 | Day 3 | Day 4 |
|  | Saturday | Sunday | Monday | Tuesday |
| 0:00 |  | Watch 2 | Watch 1 | Watch 2 |
| 1:00 |  |  |  |  |
| 2:00 |  |  |  |  |
| 3:00 |  | Watch 1 | Watch 2 | Watch 1 |
| 4:00 |  |  |  |  |
| 5:00 |  |  |  |  |
| 6:00 |  |  |  |  |
| 7:00 | 07:30 Bag Drop | Watch 2 | Watch 1 | Watch 2 |
| 8:00 |  |  |  |  |
| 9:00 | Food Team |  |  |  |
| 10:00 | Boat Call | Watch 1 | Watch 2 | Watch 1 |
| 11:00 | Briefing |  |  |  |
| 12:00 | 11:45 Dock-off |  |  |  |
| 13:00 | Start |  |  |  |
| 14:00 |  |  |  |  |
| 15:00 | Watch 1 | Watch 2 | Watch 1 | Watch 2 |
| 16:00 |  |  |  |  |
| 17:00 |  |  |  |  |
| 18:00 | Watch 2 | Watch 1 | Watch 2 | Watch 1 |
| 19:00 |  |  |  |  |
| 20:00 |  |  |  |  |
| 21:00 | Watch 1 | Watch 2 | Watch 1 | Watch 2 |
| 22:00 |  |  |  |  |
| 23:00 |  |  |  |  |